Practical 2: Specify Inputs, Outputs, State, and Interaction Contracts for CPS Components for Smart Traffic Light System

1. Aim

To design a Cyber-Physical System (CPS) abstraction for a Smart Traffic Light System by specifying the inputs, outputs, internal states, timing requirements, normal-operation contracts, and failure-handling contracts of its major components.

The three CPS components considered are:

- Vehicle/Traffic Sensor

- Traffic-Light Controller

- Traffic-Light Actuator

The practical also defines the interaction contracts between:

- Sensor → Controller

- Controller → Actuator

- Actuator → Controller

2. Objectives

The objectives of this practical are:

- To identify the inputs and outputs of CPS components.

- To define the internal states of each component.

- To represent state transitions for the sensor, controller, and actuator.

- To specify timing requirements for real-time operation.

- To define normal-operation contracts.

- To define failure-handling contracts.

- To specify the information exchanged between CPS components.

- To understand how sensing, computation, actuation, and feedback work together in a CPS.

3. CPS Scenario: Smart Traffic Light System

A Smart Traffic Light System monitors traffic at a road intersection and automatically controls traffic signals according to the detected traffic conditions.

The system uses traffic sensors to detect vehicles and estimate traffic conditions. The traffic-light controller processes this information and determines whether the signal should be RED, YELLOW, or GREEN. The traffic-light actuator receives the controller command and physically operates the corresponding traffic signal.

The basic CPS operation is:

Traffic Environment → Traffic Sensor → Traffic-Light Controller → Traffic-Light Actuator → Traffic Environment

The system is assumed to operate continuously and periodically collect traffic information.

For this practical, the following assumptions are made:

- Traffic information is sampled once every second.

- The sensor must generate a measurement within 100 ms.

- The controller must generate a decision within 50 ms.

- The actuator must execute a valid command within 100 ms.

- Emergency vehicle detection is treated as a high-priority condition.

- Invalid sensor data must not be used for normal adaptive control.

- The actuator provides feedback to the controller after executing a command.

4. Traffic Sensor Abstraction

The traffic sensor observes the physical road environment and converts traffic conditions into digital information for the controller.

4.1 Sensor: Vehicle/Traffic Sensor

Inputs

| Input | Description |
| --- | --- |
| Vehicle presence | Indicates whether vehicles are detected |
| Vehicle count | Number of vehicles detected during the sampling period |
| Sampling trigger | Indicates when a new measurement should be taken |
| Emergency status | Indicates whether an emergency vehicle is detected |
| Sensor status | Indicates whether the sensor is operational |

At least three inputs are required by the practical; five are specified here to represent the sensor more completely.

Example Input

Vehicle Count = 15
Emergency Status = FALSE
Sampling Trigger = TRUE
Sensor Status = NORMAL

4.2 Sensor Outputs

| Output | Description |
| --- | --- |
| Vehicle count | Number of detected vehicles |
| Queue length | Estimated number of vehicles waiting |
| Emergency status | Indicates emergency vehicle detection |
| Timestamp | Time at which the measurement was generated |
| Sensor status | VALID / INVALID / ERROR |

Example Output

Vehicle Count = 15
Queue Length = 8
Emergency Status = FALSE
Status = VALID

4.3 Sensor Internal States

The sensor has the following internal states:

| State | Description |
| --- | --- |
| IDLE | Waiting for a sampling request |
| SENSING | Detecting vehicles |
| VALID | Measurement successfully generated |
| INVALID | Measurement is not reliable |
| ERROR | Sensor has experienced a failure |

Thus, the sensor has more than the required four internal states.

State Transition

The sensor starts in IDLE. When a sampling trigger is received, it enters SENSING. If the measurement is successful, it enters VALID and sends the result to the controller. If the measurement cannot be trusted, it enters INVALID. Hardware or communication failure causes the sensor to enter ERROR.

An arrow will be included from INVALID → IDLE if the system retries the next measurement.

Figure 1: State-Transition Diagram of Vehicle/Traffic Sensor

4.4 Sensor Timing Requirement

The traffic sensor has a sampling period of 1 second.

The sensor must generate a measurement within:

Sensor response time ≤ 100 ms

Therefore, after receiving a valid sampling trigger, the sensor should provide the traffic measurement within 100 ms.

4.5 Sensor Normal-Operation Contract

The sensor must guarantee that:

- A measurement is generated for every valid sampling request.

- The measurement contains a timestamp.

- Vehicle count and queue information are provided in the defined format.

- Emergency vehicle detection is clearly indicated.

- Invalid measurements are marked as INVALID.

- Corrupted or incomplete data is not sent to the controller.

- A valid measurement is produced within 100 ms.

Sensor Contract

Input:

- Sampling trigger

- Vehicle presence

- Sensor status

Output:

- Vehicle count

- Queue length

- Emergency status

- Timestamp

- Validity status

Guarantee:

If the sensor is operational, a valid traffic measurement is generated within 100 ms of the sampling request.

4.6 Sensor Failure-Handling Contract

If the sensor fails or produces unreliable data:

- The sensor marks the measurement as INVALID or ERROR.

- Invalid data is not presented as a valid traffic measurement.

- The sensor status is communicated to the controller.

- The controller uses its predefined safe operating mode.

- The sensor attempts to resume normal sensing during the next valid operating cycle.

5. Traffic-Light Controller Abstraction

The traffic-light controller is the decision-making component of the CPS. It receives traffic information from the sensor, processes it, and generates a command for the traffic-light actuator.

5.1 Controller Inputs

| Input | Description |
| --- | --- |
| Vehicle count | Number of vehicles detected |
| Queue length | Number of vehicles waiting |
| Emergency status | Indicates emergency vehicle detection |
| Sensor status | Indicates whether the received data is valid |
| Timestamp | Time of the sensor measurement |
| Timing data | Current signal timing information |

Example Input

Vehicle Count = 15
Queue Length = 8
Emergency Status = FALSE
Sensor Status = VALID

5.2 Controller Outputs

| Output | Description |
| --- | --- |
| Light command | RED / YELLOW / GREEN command |
| Command timestamp | Time at which command was generated |
| Controller status | NORMAL / EMERGENCY / FAULT |
| Sequence number | Identifier for the command |

Example Output

Light Command = GREEN
Command Timestamp = 10:30:05.050
Status = NORMAL
Sequence Number = 101

5.3 Controller Internal States

The controller maintains the following states:

| State | Description |
| --- | --- |
| WAITING | Waiting for new sensor information |
| PROCESSING | Processing traffic information |
| GREEN | Green signal command is active |
| YELLOW | Yellow transition is active |
| RED | Red signal command is active |
| EMERGENCY | Emergency traffic condition is being handled |
| FAULT | Controller has detected an abnormal condition |

5.4 Controller Decision Logic

The controller uses valid traffic information to select an appropriate signal command.

Under normal conditions:

- High traffic may result in an appropriate GREEN phase within its configured limit.

- Low traffic follows the normal signal sequence.

- A GREEN phase changes to YELLOW before the next signal phase.

- The controller never generates conflicting GREEN commands.

If an emergency vehicle is detected, the controller enters the EMERGENCY state and applies the predefined emergency-control procedure.

If sensor data is INVALID or ERROR, the controller does not use that measurement for normal adaptive control and enters a predefined safe/fallback mode.

Figure 2: State-Transition Diagram of Traffic-Light Controller (via draw.io)

5.5 Controller Timing Requirement

The controller must process a valid sensor measurement and generate the required traffic-light command within:

Controller response time ≤ 50 ms

This ensures that the controller does not introduce a significant delay between sensing and control.

5.6 Controller Normal-Operation Contract

The controller must guarantee that:

- Only valid sensor measurements are used for normal control.

- Vehicle count and queue information are processed before making an adaptive decision.

- Emergency conditions are handled with higher priority.

- Conflicting GREEN commands are not generated.

- Every command contains a timestamp or sequence number.

- Signal transitions follow the predefined sequence.

- A control command is generated within 50 ms.

Controller Contract

Input:

- Vehicle count

- Queue length

- Emergency status

- Sensor status

- Timestamp

- Timing data

Output:

- Light command

- Command timestamp

- Sequence number

- Controller status

Guarantee:

For every valid traffic measurement, the controller generates the appropriate traffic-light command within 50 ms.

5.7 Controller Failure-Handling Contract

If the controller detects invalid data or an internal failure:

- It enters the FAULT or safe state.

- Adaptive traffic control is disabled.

- A predefined safe traffic-light sequence is used.

- The fault condition is recorded or communicated.

- Normal operation resumes after valid operation is restored.

The controller must not generate unpredictable traffic-light commands during a fault.

6. Traffic-Light Actuator Abstraction

The traffic-light actuator converts the digital command received from the controller into physical traffic-light operation.

6.1 Actuator Inputs

| Input | Description |
| --- | --- |
| Light command | RED / YELLOW / GREEN command |
| Command timestamp | Time when the command was generated |
| Enable signal | Allows actuator operation |
| Safety signal | Indicates whether activation is safe |
| Sequence number | Identifies the command |

Example Input

Light Command = GREEN
Enable = TRUE
Safety = TRUE
Sequence Number = 101

6.2 Actuator Outputs

| Output | Description |
| --- | --- |
| Light status | Current physical traffic-light state |
| Feedback | Indicates successful or failed execution |
| Actuator status | ON / OFF / FAULT |
| Fault indication | Indicates actuator failure |

Example Output

Light Status = GREEN
Actuator Status = ON
Feedback = SUCCESS
Fault = FALSE

6.3 Actuator Internal States

The actuator has the following states:

| State | Description |
| --- | --- |
| RED | Red traffic signal is active |
| YELLOW | Yellow traffic signal is active |
| GREEN | Green traffic signal is active |
| SWITCHING | Actuator is changing between signal states |
| FAULT | Actuator cannot safely execute the command |

:

Figure 3: State-Transition Diagram of Traffic-Light Actuator

6.4 Actuator Timing Requirement

The actuator must execute a valid traffic-light command within:

Actuator response time ≤ 100 ms

This ensures that the physical traffic signal follows the controller decision within the required time.

6.5 Actuator Normal-Operation Contract

The actuator must guarantee that:

- Only valid RED, YELLOW, or GREEN commands are accepted.

- Safety conditions are checked before activation.

- The requested traffic-light state is physically activated.

- The actual light status is reported.

- Successful execution is reported through feedback.

- Failed execution is reported as a fault.

- The command is executed within 100 ms.

Actuator Contract

Input:

- Light command

- Command timestamp

- Enable signal

- Safety signal

- Sequence number

Output:

- Light status

- Feedback

- Actuator status

- Fault indication

Guarantee:

If the actuator is enabled and the safety condition is satisfied, a valid traffic-light command is executed within 100 ms and the resulting status is reported.

6.6 Actuator Failure-Handling Contract

If the actuator cannot execute the received command:

- It enters the FAULT state.

- The physical signal is maintained or changed to the predefined safe state.

- A FAULT status is sent to the controller.

- The controller does not assume successful execution.

- The actuator remains in the fault state until recovery.

7. Interaction Contract

The interaction contract defines how the three CPS components communicate.

The practical requires three interactions:

- Sensor → Controller

- Controller → Actuator

- Actuator → Controller

7.1 Sensor → Controller

The sensor sends traffic measurements to the controller.

Data Exchanged

| Data | Description |
| --- | --- |
| Vehicle count | Number of detected vehicles |
| Queue length | Estimated waiting vehicles |
| Emergency status | Emergency vehicle detection |
| Timestamp | Measurement generation time |
| Sensor status | VALID / INVALID / ERROR |

Contract

The sensor must provide valid and timestamped traffic information to the controller within 100 ms of the sampling request.

If the measurement is invalid, the sensor must clearly indicate its INVALID or ERROR status.

The controller must not use invalid measurements for normal adaptive control.

7.2 Controller → Actuator

The controller sends traffic-light commands to the actuator.

Data Exchanged

| Data | Description |
| --- | --- |
| Light command | RED / YELLOW / GREEN |
| Timestamp | Time command was generated |
| Sequence number | Command identifier |
| Controller status | NORMAL / EMERGENCY / FAULT |

Contract

The controller must generate a valid traffic-light command within 50 ms for every valid sensor measurement.

The actuator accepts the command only if it satisfies the required validity and safety conditions.

7.3 Actuator → Controller

The actuator sends execution feedback to the controller.

Data Exchanged

| Data | Description |
| --- | --- |
| Light status | Actual active signal |
| Feedback | SUCCESS / FAILED |
| Actuator status | ON / OFF / FAULT |
| Fault indication | Indicates execution failure |

Contract

The actuator must report the actual physical signal status after executing the command.

If execution fails, the actuator must send a FAULT indication to the controller so that the controller can enter the appropriate fault-handling state.

8. Component Contract Summary

| Component | Inputs | Outputs | Internal States | Timing |
| --- | --- | --- | --- | --- |
| Traffic Sensor | Vehicle presence, vehicle count, sampling trigger, emergency status | Vehicle count, queue length, emergency status, timestamp, sensor status | IDLE, SENSING, VALID, INVALID, ERROR | ≤ 100 ms |
| Controller | Vehicle count, queue length, emergency status, sensor status, timestamp | RED/YELLOW/GREEN command, timestamp, sequence number, status | WAITING, PROCESSING, GREEN, YELLOW, RED, EMERGENCY, FAULT | ≤ 50 ms |
| Actuator | Light command, timestamp, enable, safety | Light status, feedback, actuator status, fault | RED, YELLOW, GREEN, SWITCHING, FAULT | ≤ 100 ms |

Table 1: Component Contract Summary

9. Timing and End-to-End Requirement

The assumed timing requirements are:

- Sensor response ≤ 100 ms

- Controller response ≤ 50 ms

- Actuator response ≤ 100 ms

Therefore, the maximum approximate processing time from traffic detection to physical signal execution is:

100 + 50 + 100 = 250 ms

Thus:

The Smart Traffic Light System should respond to a detected traffic condition within approximately 250 ms, assuming communication delays are negligible.

The sensor sampling period is assumed to be 1 second.

10. Overall Failure-Handling Contract

The system must continue to operate safely even when one of its components fails.

- Sensor Failure

If the sensor reports an ERROR or INVALID condition:

Sensor → INVALID/ERROR → Controller detects failure → Safe/Fallback operation

The controller must not make adaptive decisions using invalid sensor data.

- Controller Failure

If the controller stops generating commands, the actuator must not continue indefinitely using an old command. A watchdog mechanism may be used to detect command timeout and place the actuator in a predefined safe state.

- Actuator Failure

If the actuator cannot execute a command:

Controller → Command → Actuator → FAULT Feedback → Controller

The controller must recognize that the physical signal did not change successfully and enter the appropriate fault-handling state.

11. Practical Results

The Smart Traffic Light System was modeled using three CPS abstractions:

- Vehicle/Traffic Sensor

- Traffic-Light Controller

- Traffic-Light Actuator

Each component was specified with its inputs, outputs, internal states, timing requirements, normal-operation contract, and failure-handling contract.

The interaction contracts between the sensor and controller, controller and actuator, and actuator and controller were also defined.

The timing analysis gives a maximum approximate end-to-end response time of:

250 ms

The state-transition diagrams show how each component responds to normal events and failure conditions.

12. Conclusion

The Smart Traffic Light System demonstrates the interaction between sensing, computation, and physical actuation in a Cyber-Physical System.

The traffic sensor observes vehicle and traffic conditions and sends the resulting information to the controller. The controller processes the information and generates a RED, YELLOW, or GREEN command. The actuator converts the command into a physical traffic-light state and sends feedback to the controller.

The component contracts define the expected inputs, outputs, timing, and behaviour of each component, while the interaction contracts define how information is exchanged between them.

The inclusion of failure-handling contracts ensures that invalid sensor information, controller failures, and actuator failures do not result in uncontrolled operation. Therefore, the abstraction provides a simple model of a reliable and time-constrained Smart Traffic Light CPS.